Assignment 4

Troubleshooting And Helpdesk

Section 1: Multiple Choice

- What is the first step in the troubleshooting process?

  - Implementing a solution

  - Identifying the problem

  - Testing the solution

  - Documenting the solution

Ans. -> B. Identifying the problem

- Which of the following tools is commonly used to diagnose hardware issues by testing electrical connections?

  - Loopback plug

  - Toner probe

  - Multimeter

  - Cable tester

Ans. -> C. Multimeter

- Which Windows utility can be used to view system logs, monitor performance, and diagnose hardware and software issues?

  - Task Manager

  - Device Manager

  - Event Viewer

  - Control Panel

Ans. ->C.  Event Viewer

Section 2: True or False

- True or False: Safe Mode is a diagnostic mode in Windows that loads only essential system services and drivers, allowing users to troubleshoot and fix problems with the operating system.

Ans. -> True

- True or False: A system restore point is a snapshot of the computer's system files, registry, and configuration settings at a specific point in time, which can be used to revert the system to a previous state if problems occur.

Ans. -> True

- True or False: Ping is a command-line utility used to test network connectivity by sending ICMP echo requests to a target device and waiting for ICMP echo replies.

Ans. -> True

Section 3: Short Answer

- Describe the steps involved in troubleshooting a computer that fails to boot into the operating system.

Ans. -> Steps involved in troubleshooting a computer that fails to boot into the operating system are given below:

- Check the computer is receiving power supply correctly and ensure are cables and components of computer system is connected properly.

- Now disconnect the peripherals devices like USB drives, external disks and any other which should affect the computer system in booting.

- Restart the computer and enter BIOS menu.

- Check hard drive is detected.

- Check the boot order if it is not correct then set it correct.

- If not worked then reset BIOS to default settings.

- If the computer power-on self-test (POST) finishes but Windows fails to load, force-restart three times to trigger automatic repair.

  - Select Troubleshoot > Advanced Options > Startup Repair.

  - Attempt to boot into Safe Mode to rule out driver issues.

- If software fixes fail, shut down the computer and open the cabinet for:

  - Reset RAM modules

  - Check hard drives cables are connected properly

  - Check for the signs or evidence of overheating or component damage.

- If boot sector or OS is corrupted then reinstall it using bootable device for repairing or cleaning the OS.

Section 4: Practical Application

- Demonstrate how to troubleshoot network connectivity issues on a Windows computer using the ipconfig command.

Ans. -> Steps to troubleshoot network connectivity issues using the ipconfig command (Windows):

  - Press win + R and type cmd and press Enter to open command prompt or simply type command prompt from search menu.

  - Type ipconfig and press Enter to see the IP address

  - Check the network has a correct IP address

  - Run ipconfig /all to see full details, including MAC address, DHCP server and DNS sever addresses

  - If there is no connect, use ipconfig /release to drop all the current IP address and then type ipconfig /renew for requesting a new IP address from the router.

  - Run ipconfig to confirm a valid IP is assigned.

  - If websites fail to load, run ipconfig /flushdns to clear the DNS cache, which resolves outdated or corrupted website address information.

  - Use ipconfig /all | findstr /c:"DNS Servers" to identify your configured DNS server and troubleshoot name resolution issues.

Section 5: Essay

- Discuss the importance of effective communication skills in a helpdesk or technical support role.

Ans. -> Effective communication is often considered as critical as technical expert in helpdesk and support roles, as it serves as the primary bridge between complex technology and end-users.

- Reason for effective communication skills

- Active listening allows support agents to accurately understand issues by collecting or noticing all necessary details before jumping to a solution.

- Supported roles require the ability to explain complex and technical in simple so non-technical users can follow troubleshooting steps without confusion.

- When customers feel heard and understood, it builds confidence in the support provided. Research shows 96% of customers view service as a key factor in brand loyalty. So it also gives satisfaction to the customer and the review will also be good.

- Clear and simple instructions reduce repeated questions and messages, which helps solve problems faster and reduces system downtime.

- Support agents often deal with frustrated users. Empathy and professional communication can calm tense situations and turn a negative experience into a positive one.

- Communication skills are also important for documenting issues and solutions accurately, so that future problems can be resolved faster and knowledge can be shared among team members.

- Effective communication helps users understand what caused the problem and how to avoid it in the future.

- Good communication reduces errors during remote troubleshooting.

- Honest communication about issue status builds transparency and trust.

- Confirming user understanding ensures the solution is applied correctly.

- Proper questioning helps gather complete system and error information.

- Reassuring communication reduces user anxiety during system failures.

- Clear follow-up messages ensure the issue is fully resolved.

- Good communication prevents repeated support requests for the same issue.

- Positive interaction increases user cooperation during troubleshooting.

- Clear explanations help users feel more confident using technology.